Setting up Blinky LED on STM32 NUCLEO-F446RE

First we have the board ready with us - NUCLEO-F446RE

Then we installed the following softwares:

i) STM32CubeMX Version: 6.17.0

https://www.st.com/content/st_com/en/stm32cubemx.html#st-get-software

ii) STM32CubeIDE

Version: 2.1.0

Build: 27993_20260219_1630 (UTC)

https://www.st.com/en/development-tools/stm32cubeide.html

After installing both, The we opened the STM32CubeMX application

- File->New Project

- Under Board Selector, need to type “NUCLEO-F446RE” and click on Start Project and then click on yes to Initialize all the peripherals to default value.

- Make sure to check that 39/50 GPIOs pin unselected and under Project Manager tab, choose STM32CubeIDE instead of EWARM.

- Ctrl+S-> Save the project location and then click on “Generate Code”

- IOC File will be generated

- Open STM32CubeIDE application and then import STM32CubeMX project

- Select the parent directory where ioc file is present and then the project with all the files will get available on the left hand side.

- Go to Core->Src->main.c file

- Inside main.c file search for while(1) loop where user has to write the code.

- Enter the desired code for blinking the LEd at LD2 pin with 1 second interval.

HAL_GPIO_TogglePin(GPIOA, GPIO_PIN_5);

HAL_Delay(1000);

- Go to Project->Clean and then Build

- Connect the Hardware and make sure the lights are ON.

- Go to Help->ST-LINK Upgrade and then click on “Open in Update Mode” and then click on Upgrade.

- After the Upgrade successful message comes, click on the Run->Run As->STM32 C/C++ Application

- The LED will blink red and gree present at the top of the board making sure that the flashing is taking place.

- Once flashing is done, then automatically the LED on LD2 blinks ta 1 second interval.